Hydrological observations in a micro-scale rainforest catchment in Central Amazonia, Brazil.

J.M. Heijmenberg

Amsterdam, April 2002

Hydrological observations in a micro-scale rainforest catchment in Central Amazonia, Brazil.

J.M. Heijmenberg

Amsterdam, April 2002

Table of Contents

1	Introduction	5

2	Site Characteristics	7

3	Methods	9

3.1	Saturated hydraulic conductivity	9

3.2	Surface runoff	10

3.3	Dissolved organic carbon and nutrient losses in streamflow	11

4	Results and discussion	13

4.1	Saturated hydraulic conductivity	13

4.2	Surface runoff	17

4.3	Dissolved organic carbon and nutrient loss in stream flow	22

5	Conclusions and recommendations	25

5.1	Hydraulic saturated conductivity	25

5.2	Surface runoff	25

5.3	Dissolved organic carbon and nutrient loss in stream flow	25

Table of figures

Figure 1 	Location INPA reserve Northern	6

Figure 2	Location of the Cuieiras catchment 50 kms North of Manaus in the INPA reserve	6

Figure 3	Map of the Cuieiras catchment showing the topography, roads, discharge measurement and selected groundwater measurement sites.kaolinite.	7

Figure 4	The Hooghoudt auger hole method	9

Figure 5	Wavelength regions of the different compounds (Time-resolved spectrometry for water management applications, case studies and suggestions, S::can)	12

Figure 6	Saturated hydraulic conductivity Cuieiras valley	13

Figure 7	Different zones of Ks in the Cuieiras valley	14

Figure 8 	Groundwater contribution of the different zones in the valley	15

Figure 9	Spatial distribution of the saturated hydraulic conductivity of the valley.	15

Figure 10	Extra transect Ks	16

Figure 11	Q-h relation, calibration equation  y = 1E-05x2,3373 R2 = 0,9954	17

Figure 12	Wet cross section in at discharge measuring site, h=65cm	18

Figure 13	Calculated surface areas of the different segments	19

Figure 14	Velocity profile at h=65 cm	19

Figure 15	Cross section at h=107	20

Figure 16	Hydrograph Cuieiras catchment 9-12-01 till 26-2-02	21

Figure 17	Contributing area to the discharge of the Cuieiras catchment	21

Figure 18	Specific absorption spectrum for groundwater	23

Figure 19	Detail of the river water spctrum from 3:00 hrs, the rain starts after a dry period, the absorption becomes higher for the carbon range.	24

# Introduction

This report presents the results of a short hydrological study carried out by Mr. Joost M. Heijmenberg as part of his MSc. Programme at the Faculty of Earth and Life Sciences, Vrije Universiteit Amsterdam. The research was carried out in January and February 2002 in the Central Amazonian Cuieiras rainforest Reserve of the Instituto Nacional de Pesquisas Amazônicos (INPA) in Manaus, Brazil. The study was supervised by Dr. Maarten J. Waterloo from the Vrije Universiteit Amsterdam and Dr. Antonio D. Nobre from INPA. The study was carried out within the framework of the EU funded LBA Carbonsink project under the expedition license of the CNPq-MCT Carbo-Amazonas project.

Micrometeorological measurements of carbon gains and losses to the atmosphere in the gaseous phase (i.e. CO2 fluxes) suggest that the forest has acted as a sink for carbon over the past three years (Araújo et al., 2002). To obtain more accurate knowledge on the magnitude of this sink, gains and losses through other mechanisms need to be quantified. These mechanisms include releases and uptake of Volatile Organic Compounds (VOCs) and trace gasses (e.g. methane) as well as gains and losses from dissolved and particulate carbon in water entering and leaving the forest ecosystem. The purpose of the hydrological research in the Carbo-Amazonas project is therefore to quantify the fluxes of dissolved and particulate carbon in different compartments of the hydrological cycle and to study the processes leading to carbon leaching.

The goals of the present study were:

- Determination of the saturated hydraulic conductivity of valley soils in a micro-scale catchment to provide data required for modelling losses of carbon associated with groundwater flow.

- Developing a discharge rating curve for the stream (locally called Igarapé) and determine correction factors for calculating discharge from flow velocity measurements to calculate total discharge from December 2001 until February 2002.

- Characterisation of different water types according to their spectral absorption and Dissolved Organic Carbon (DOC) concentrations, acidity and electrical conductivity to provide an estimate of the carbon loss associated with streamflow out of the catchment.

The location of the research area is shown in Figure 1 and Figure 2, the latter being a satellite image of the area, which also shows the city of Manaus and the degree of deforestation around the city and along the BR-174 highway towards Caracas, Venezuela (red-pink and light-green colours).

Detail

Figure 1. Location of the city of Manaus and the INPA reserve in Central Amazonia.

Cuieiras Catchment

Manaus

Figure 2. Landsat satelite image showing the location of the Cuieiras catchment, about 50 km north of Manaus in the INPA forest reserve.

# Site Characteristics

The Cuieiras micro-scale catchment (2°36’S, 60°12’W, area 6.37 km2) is located 50 km to the north of Manaus (Figure 1 and Figure 2) and is covered by undisturbed rainforest. The area drains towards the Cuieiras River in the west and includes five small headwater catchments draining surrounding plateau areas. The plateau areas vary in height between 90 and 110 m a.s.l and are strongly incised with steep, concave slopes leading to rather broad, flat valley bottoms at 40-60 m a.s.l. The river at the discharge measurement site is at about 46 m a.s.l. and the micrometeorological tower on the plateau at 91 m a.s.l. The camp (Moto da Linea) is at 64 m a.s.l. A map of the area is shown in  .

Figure 3. Map of the Cuieiras catchment showing the topography, roads, discharge measurement and selected groundwater measurement sites.

The well-drained soils on the plateau and slopes consist of a yellowish sandy loam with little variation in colour of the surface horizons and are classified as Podzolica vermelho-amarelo in the Brazilian soil classification system. Colour changes occur deeper in the profile as a result of past variations in the water table depth (presently at around 37 m below the surface on the plateau). The valleys are swampy, with water levels close to the surface (at less than 30 cm depth) even during the dry season. These poorly-drained valley soils are distinctly different from the soils on the slopes and plateaus and consist of coarse to fine quartz sand and contain significant amounts of fine, dark-coloured organic matter particles. Soil profiles on the foot slopes, where the groundwater level is at about 1 m depth, show a transition from a yellowish/orange loamy sand near the surface to bright white clayey sand in the saturated zone. The bright white colour seems to be derived from the presence of kaolinite clay. A layer containing coarse quartz sand and gravel has been observed on the transition from the unsaturated to the saturated zone.

The vegetation shows considerable spatial variation in relation to topography and drainage status of the soil. Terra firme (dry) forest is found in the well drained parts of the area (slopes and plateau), whereas more open, Campinarana forest is found on the transition area between well-drained and poorly drained soils and swamp forest containing many palms is found in the saturated zones of the catchment (e.g. valleys).

# Methods

## Saturated hydraulic conductivity

The saturated hydraulic conductivity (Ks, in m d-1) of the valley soils was determined with the Hooghoudt auger hole method. The Hooghoudt set consists of a special filter tube, a pulse auger, a float and a stopwatch. The filter is placed into the (saturated) soil at a maximum depth of 2 m using a specially designed auger, after which water is removed from the filter/borehole with an electrical pump. A float is then inserted into the filter tube and the rate of change in the groundwater level is recorded using the float movement and a stopwatch. The rate of change in the water level, together with the geometry of the filter provides an indication of the saturated hydraulic conductivity of the soil around the filter in the borehole. The following parameters are of importance when measuring Ks (see also Figure 4)

Z	= Measuring tape reading (in cm) at normal ground level

H	= Depth of the auger hole (in cm) below ground water level

y0	= Water level in the auger hole at the beginning of the measurement (in cm) below normal groundwater level.

yn	= Water level in the auger hole during measurement operations.

y	= Rise of water level in auger hole during the measurement.

y	= 0.5 (yn + y0) = average water level during the measurement.

2r	= Auger hole diameter (in cm).

Figure 4. Diagram showing important parameters for Ks calculation using the Hooghoudt auger hole method.

The following formulae can then be used for calculating the value of Ks (Ernst, 1950):

Ks = 	4000 r2	 / (H + 20r)*(2-y/H) * y/t

In order to get an indication of the spatial variation of Ks in the valley, several transects were laid out along which measurements were made. These transects are parallel and in the same NNE-SSW direction as that of the groundwater access tubes (piezometers). Three transects were laid out spaced 50 meters apart with measurement points along these transects being spaced 30 m apart.

## Surface runoff

Due to the particular shape of the valleys and the coarse valley soil texture, traditional methods of measuring discharge (weirs, flumes, etc.) were not likely to work well. A weir may cause a large area upstream to remain inundated, whereas the height-discharge relationship for flumes may not be unique due to backwater effects. As such, a different approach has been taken to measure the surface water discharge. The absolute value of the river water level was regularly measured with fixed staff gauges. Doppler streamflow velocity sensors were installed to continuously measure streamflow velocity and depth of the river. Two types of ultrasonic flow meters (Australian Starflow doppler sensors (UNIDATA) and VTM1 doppler sensors (RMI, USA)) were used. The two Starflow systems were installed in the river channel, whereas the two VMT1 flow meters were installed in the floodplain. The latter therefore only record data under bank overflow conditions. Two Starflow flow meters are installed on small metal platforms placed at the bottom of the stream channel. Both instruments had a logging time interval of 10 minutes. The velocity data did not give representative data for the average stream velocity. For this reason only the digital depth of the Starflow was used. In order to find a representative relation between the discharge (Q) and the water level (h) two staff gauges were installed. The top of the lowermost of these gauges represents the topographic reference to which all other instrument heights have to be related. This staff gauge is located in the river, whereas the other one is higher up in the floodplain. Together, these gauges provide the means to measure water levels of up to 2 m above the river bottom. These staff gauges also provide the water level height reference to which the continuous measurements (see below) were referenced. A discharge-rating curve has been derived from salt dilution discharge measurements at different discharges near the location of the doppler sensors and flow velocity measurements. The Q-h relation:

Q = a * (w-zp)b

In this formula Q is the discharge, w is the gauge level, zp is the zero point (i.e. the gauge level which corresponds to the lowest point of the riverbed cross section), a and b are parameters derived from the trend line of the log values of Q. A Q-h relation could be made for values of w of up to 94 cm. For values higher than 94cm, bank overflow occurs and the rather flat and broad floodplain becomes inundated. At this stage, salt discharge measurements did not work. For this reason discharge measurements were done using the velocity-area method. This method consists of measurements of measurements of stream velocity, depth of flow and distance across the channel between observation verticals. For these measurements a current meter (Ott Messtechnik, Germany) was used to measure velocity-depth profiles through a cross-section of the channel and floodplain. The cross-section micro-topography of the river and floodplain (i.e. the micro-topography of along a transect perpendicular to the flow direction at the flow meter station) was measured in detail. This topographic profile provides information on the cross-section area (wet cross section) at specific water levels, which is used in combination with the velocity depth profiles to calculate the discharge. This technique was also used for lower water levels to check for consistency with the results from the salt dilution method and Starflow Doppler sensors.

## Dissolved organic carbon and nutrient losses in streamflow

A UV-Vis spectrometer (S::can UV-Vis spectrolyser) was used to obtain continuous time series of concentrations of DOC and NO3. The S::can spectrolyser measures the absorption of radiation in wavelengths in the UV-VIS (for visual) range, i.e. 200 – 750 nm. The probe consists of an UV-Vis light source and an array of 256 cells which measure radiation at intervals of 3 nm wavelength. The measurements of these cells provide an absorption spectrum over the full range of the UV-Vis wavelengths. The measurement is differential, which means that the beam of the UV-Vis source through a closed a path in the instrument (no absorption) is compared to a second beam which goes through the sample. As such any effects of drift in the UV-Vis source on the sample analyses are avoided. The instrument can measure NO3 (absorbs at 215-230 nm), dissolved organic compounds (absorption in a wide range of wavelengths but well-correlated with absorption at 254 nm or at 450 nm), turbidity (attenuation of the light in the 700 nm wavelength) and a range of other specific organic substances including benzene (Figure 5).

Figure 5. Wavelength regions of the different compounds (Time-resolved spectrometry for water management applications, case studies and suggestions, S::can information sheet)

A local DOC calibration equation was used to calculate DOC from the absorption at 254 nm:

DOC = 0.3373  - 0.5699					R2 = 0.9765

where DOC is the dissolved organic carbon (mg l-1) and  the absorption (m-1) at a wavelength of 254 nm.

The probe was placed on a support in the river at the outlet of the catchment and measured streamwater spectra at intervals of 15 minutes. In addition, rainfall, throughfall, litter percolate and groundwater samples were collected for determination of their spectra. Rainfall was collected on the top of three towers, whereas throughfall and litter percolate were collected at 12 points along a transect running from the plateau down to the valley. A total of 24 throughfall gauges and 12 litter percolate collectors were installed for this purpose. Groundwater was collected from wells placed along a similar transect with water levels in these wells varying between less than 0.1 m (valley) and 37 m (plateau) depth below the surface.

# Results and discussion

## Saturated hydraulic conductivity

A total of 30 Ks measurements were made along the transects described in the previous section. Each transect started in the Southern part of the valley and the southernmost measurement points were located in the Campinarana transition zone. In this zone, the groundwater level is at about 1 m depth, which is close to the depth limit for Ks measurements with the Hooghoudt set. Installing the filter in the coarse sandy soils of the Campinarana forest was difficult, as the soil tended to fill the auger hole before the filter could be lowered. Furthermore, the quick response of the groundwater level due to the high permeability of the coarse sandy soils made it difficult to do accurate measurements. The location of first measurement point is indicated at zero distance in Figure 6 and from this point onwards 9 measurement points were selected at 30-meter intervals. The value of Ks in the river was set to zero to provide a visual indication of the location of the river in the plots.

Figure 6. Saturated hydraulic conductivity of the valley near the discharge measurement point .

Three zones with different Ks are distinguished in the valley cross sections of Figure 6:

- The river, where Ks was set to zero

- The floodplain area with generally low Ks values

- The southern hill slope area, bordering the Campinarana zone with higher Ks values

- Northern hill slope area, which does not have Campinarana forest, but which also shows an increase in Ks as compared to the floodplain.

The areas are indicated the valley cross section below (see Figure 7)

Figure 7. Different zones of Ks in the Cuieiras valley.

The soil in the floodplain consists of fluvial deposits. These sediments consist of a mixture of clay, sand, gravel and fine organic particles. Local changes in soil texture can be explained by the continuously changing position of the river bedding in time. Presumably for this reason, unit B shows a high spatial variation of Ks, related to the local texture of the material In spite of the high spatial variability for Ks in the floodplain, a general trend can be seen for the valley cross-section. The measurements generally show increasing Ks values in the direction of the hill slope. The highest values are found in the Campinarana zone (up to 6 mm d-1). This zone, with a width of about 200 m, is found adjacent to the southern hill slope. The soil of this zone consists of coarse sediment with pure-quartz gravel particles up to 1.5 cm diameter. The undergrowth vegetation is less dense than in the floodplain. The saturated hydraulic conductivity of the soils near the river is low (about 0.5 mm/24 hrs) due to clay deposits. Higher values near the hill slopes are caused by the coarser texture of the soils near the hill, due to slope debris and older fluvial deposits. In Figure 8, the relative influence of the saturated hydraulic conductivity on the groundwater contribution is indicated. Both hill slopes provide lateral groundwater contributions to the floodplain. During dry periods this process may sustain the base flow. The Campinarana forest zone acts as an infiltration area. During rainfall events, water will move through this zone very fast (pers. Comm. M. Hodnett) and can therefore be transported rapidly to the floodplain.

During dry periods, only a small amount of water is moved towards the stream from the floodplain itself. During rainfall events, the soil becomes saturated very fast as a result of the high groundwater water levels (even during the dry period). This causes saturated overland flow in the floodplain towards the stream, resulting in a quick repsonse of the stream water level to rainfall.

Figure 8 	Groundwater contribution of the different zones in the valley

The following contour map (see Figure 9) shows the spatial distribution of the saturated hydraulic conductivity of the valley soil. The average trend in the spatial distribution becomes clear. An extra transect was set out in order to check this distribution of Ks. Figure 10 shows the values for Ks measured in this transect.

Figure 9	Spatial distribution of the saturated hydraulic conductivity of the valley.

Figure 10. Extra transect Ks.

## Surface runoff

Discharge measurements at high water levels were combined with measurements during the dry season. Eventually a reprehensive Q-h relation could be extracted.

Figure 11 shows the Q-h relation for values up to H=94 cm. From this point upwards, bank overflow occurs and the salt dilution method fails. Manual velocity-depth profile  measurements indicate that the discharge value for these high water levels do not fit the Q-h relation made for the lower values. As such, a second Q-h relation was derived from the manually discharge measurement during high discharges.

Figure 11	Q-h relation, calibration equation  y = 1E-05x2,3373 R2 = 0,9954

To obtain a Q-H relation for values of H higher than 94 cm, the velocity-area method was used. Figure 12 shows the wetted river cross-section at a water level of 65 cm.

Figure 12	Wet cross section in at discharge measuring site, h=65cm

To calculate discharge, the mid-section method was used. In the mid-section method of computation it is assumed that the sampled at each vertical represents the mean velocity in a segment. The segments area shown in Figure 13. The segment area extends laterally the mean velocity. The segment discharge is then computed for each segment and these are summed to obtain the total discharge Figure 14 shows the average velocities for different segments averaged for successive depths.

Figure 13	Calculated surface areas of the different segments

Figure 14	Velocity profile at h=65 cm

The velocity area method was also used for high water levels. The topography of the valley was measured (Figure 15) and the wet cross section (Figure 16) was made for a water level at h=107. As described above, a velocity profile was measured using the odd and the discharge was calculated. The discharge at h=107 was 1.8 m3/s.

Figure 15	Topography of the valley

Figure 16	Cross section at h=107

Using the stage-discharge relation a hydrograph could be established using the data from Starflow II (Figure 17).

Figure 17	Hydrograph Cuieiras catchment 9-12-01 till 26-2-02

The total amount of water leaving the catchment is:  2087319 m3/s    The catchment area  was 6.37 km2 (Figure 18), leading to a total discharge of 328 mm in the period of 9 December until 26 February.

Figure 18	Contributing area to the discharge of the Cuieiras catchment

## Dissolved organic carbon and nutrient loss in stream flow

To characterise the different water types according to their spectral absorption, averages of different samples were taken. Figure 19 shows the average specific absorption of the different water types. The UV-Vis spectra of rainfall, throughfall, litter percolate, stormflow, baseflow and groundwater clearly show that the water in the valley is rich in organic matter, in contrast to that of rainfall, deep groundwater under the plateaus and, to a lesser extent, of throughfall. The stream also shows contrasting spectra for stormflow and baseflow conditions due to the influx of litter water during the former The text below will describe the different water types according to their absorption in detail.

Figure 19	Characterisation of different water types according to their spectral absorption

Four samples of each type of water were collected and analysed. No significant difference in absorption was found, except for the groundwater. If we look to the absorption spectrums of the different groundwater samples (Figure 20) we see two groups of absorption spectrums.

Figure 20	Specific absorption spectrum for groundwater

Starting at the highest point and going down towards the floodplain, the groundwater is comparable for al the samples. The groundwater does nearly give no absorption and is almost comparable with the spectrum of rainwater (Figure 19). For piezometer  T4 and those in the floodplain, different absorption spectra were found. The groundwater spectrum shows peaks in the organic carbon range and is similar to the spectrum of litter water (Figure 19). This indicates that the groundwater in the floodplain is almost comparable with the litter water and already has a high carbon concentration.

The litter water also shows a high absorption in the carbon range. This is due to the leaching of dissolved carbon from the litter in the decomposition process. Water percolates through the litter layer and removes away organic material. When the TOC is calculated using the absorption at 255 nm, values of up to 25 mg/l were found. The litter water spectra apparently did not change with elevation in the catchment, although this was hard to measure because the concentration also depends on the amount of litter on the gauge.

River water shows a variation in spectra, which depends on the amount of rainwater/litter water that flows into the stream. After a rainfall event we see the absorption going up in the carbon range. This may represent a fast transfer of litter water to the the river. During rainfall events saturated overland flow washes away a great amount of organic material out of the floodplain which results in an extra contribution of dissolved carbon in the river water. During dry periods the river is mainly fed by groundwater with a lower amount of carbon. In Figure 21 a detail of the absorption spectrum is shown during a rainfall event. The absorption in the carbon range is going up during the rain event and goes down afterwards.

Figure 21	Detail of the river water spectrum from 3:00 hrs, the rain starts after a dry period, the absorption becomes higher for the carbon range.

Throughfall only shows little absorption in the carbon region and its characteristic spectrum lie between just above those of rainwater

For making an estimation of the total amount of dissolved organic carbon that is leaving the system, an average concentration was taken as data were available for a few days only. The average TOC that is leaving the catchment is about 20 mg/l. combining this value with the total amount of water that is leaving the system (4.2 p.17) gives  a value of 41,000 kg for the total export of carbon from the catchment between 9 December 2001 and 26 February 2002, or 66 kg ha-1.

# Conclusions and recommendations

## Hydraulic saturated conductivity

In general, Ks is high in most of the valley area (average of 3 mm d-1), although low values can be found along the stream due to the local clay deposits (0.5 mm d-1). The sandy Campinarana zone on the foot slopes shows the highest values. This zone must therefore be considered as an important infiltration zone for sustaining baseflow during dry periods. Runoff water from the higher, Northern hill slope infiltrates in this zone and is quick transported to the lower floodplain. The fact that the valley soils remain saturated for most of the time explains the quick response of the stream to rainfall.

Results obtained from the Ks measurements mostly show considerable spatial variation, even when the measuring points are not too far apart. This is may be due to changes in texture, as well as in structure due to biological activity. However the range of Ks remains rather small (1-6 mm d-1). Additional measurements are recommended during the dry season, in order to improve upon the present estimates.

## Surface runoff

Using the depth data from Starflow II a precise stage-discharge relation could established for values up to h=94 cm. From this point upward, more velocity – depth measurements should be made with a current meter to construct a discharge stage relation for high stages.

## Dissolved organic carbon and nutrient loss in stream flow

Water analysed by the Scan probe show clear differences in absorption. Using these characterisations the contribution of different water types towards the stream flow at different moisture conditions can be detected. The UV-Vis spectra of rainfall, throughfall, litter percolate, stormflow, baseflow and groundwater clearly show that the water in the valley is rich in organic matter, in contrast to that of rainfall, deep groundwater under the plateaus and, to a lesser extent, of throughfall. The streamwater also shows contrasting spectra for stormflow and baseflow conditions due to the influx of litter water during the former.

During periods of high water, the river water shows high peaks in the carbon range and its chemical composition is comparable to that of litter percolate. Because the present study ended in March, only a short time serie could be of river water composition could be studied. Additional measurements are made over a longer period to monitor the outflow of carbon. A detailed analysis of the carbon range spectrum is recommended to find out the contribution species leaving the system.

As the absorbance at 254nm is out of the measurement range it would be better to use a smaller path length or to use higher wavelengths for calibrating the TOC.

References

Waterloo, M., Carbon cycle Amazonas Manual for hydrological measurements in the Cuieiras reserve of INPA, Alterra, Vrije Universiteit, Amsterdam, 2001

Reginald, W.H., Streamflow Measurement, Partner, CNS Scientific & Engineering Services Reading, UK, 1995

Lance, F and Lesack, W., Water Balance and Hydrologic Characteristics of a Rain Forest Catchment in the Central Amazon Basin, Department of biological Sciences, University of California, Santa Barbara, 1993

Lance, F and Lesack, Export of Nutrients and Major Ionic Solutes from a Rain Forest catchment in the central Amazon Basin, Department of biological Sciences, University of California, Santa Barbara,

Eijkelkamp, Equipment for determing water permeability of a soil in situ using the auger hole method, Giesbeek, 1983

for S>= 0.5H,  S>H, y<= 0.25 y0  and y> 0.2H

See 4.1 Saturated hydraulic conductivity, p.13

for location of the piezometers see Appendix